Identification of Reasoning Errors

This document is restricted to be used by the “Students” who have attended the Error-Free AI Enhanced Multi-pass Technical Review training course, offered by Error-Free Inc. It is copyrighted by Error Free Inc. and Performance Improvement International, USA, 2025. It shall not be revised, copied, or re-compiled to generate derivative products for any purpose, especially in forms of derivative products to be included in a company procedure or training materials for use by non-students.

The purpose of this document is to provide a six-step guideline to identify Reasoning errors in a review document, as well as its risk level, the method that finds errors, and the recommended corrections.

Definitions of Contents in the Review Document

In general, the content of a review document for a specific purpose could be divided into six categories: requirements, scope of work, method, input, output, and testing. Each of these categories is described as follows.

- Requirements are defined as the rules specified in the document that the remaining content must comply with to support the purpose of the document. These rules include, but are not limited to, regulatory regulations and laws, specifications in industry codes and standards, contractual terms and conditions, technical specifications, operating specifications, design specifications, and company procedures and policies.

- Scope of Work is defined as the necessary activities, functions, operating ranges, conditions for analysis, etc., the product defined in the document to support all the requirements.

- Method, also known as methodology, is defined as a means of synthesizing input to produce the desired output based on stated and intended requirements. The method may include analysis, calculation, logical reasoning, sequencing activities for planning, sequencing actions for procedures, and more.

- Input to the method described in the document is defined as parameters that are necessary for the method to generate output. Input information consists of five categories of data:

  - Technical data (such as material properties, dimensions, analysis results from another work, etc.)

  - Environmental data (as such temperature, pressure, weather, etc.)

  - Action or activity (such as turning on a pump, locking a switch, etc.)

  - Reactions from shareholders (such as expected activities from the affected parties, etc.)

  - Supply data (such as manpower into an activity, spare parts, etc.)

- Output is defined as the results of an analysis, a work, or a production process.

- Acceptance testing, also known as post-work testing, is defined as tests that validate and verify (V&V) the produced results. The acceptance testing for a product consists of testing the product of all its design functions and LOPs in all expected operating conditions, which consists of normal operating conditions, abnormal conditions, and trip conditions. The acceptance criteria for each test should be clearly specified.

Identification Steps to Find Reasoning Errors

Step 1: Identify Possible Types of Events that Could be Caused by the Failures of the Review Document

The possible types of events could be:

- Failure of the equipment described in the document.

- Failure of other equipment connected to the equipment described in the document.

- Defeat of a safety system, such as a trip mechanism

- Failure of the intended purpose of the review document

- Fire

- Injury

- Others

Step 2: Based on the Types of the Events Identified in Step 1, Identify the Possible Failure Modes

Determine the failure modes that could cause the events identified in Step 1. These failure modes would be used to determine the critical work that], if it fails, it could cause significant consequences.

Ste 3: Using Commission Error Identification Methods to Identify the Reasoning Errors According to the Types of the Errors

Reasoning errors are commission errors. There are 34 types of reasoning errors as described in Attachment 1. There are 39 identification methods to find commission errors, as described in attachment 2. The review document shall be reviewed to find each type of reasoning errors using all 39 commission error identification methods.

Step 4: Determine the Risk of Each Reasoning Error

The evaluation shall be performed by estimating the possible level of consequence (High, Medium, and Low) of the identified Reasoning error based on the failure modes and events identified in Step 1 and 2. The level of consequence is defined as follows:

Low Level of consequence: Causing a non-compliance event, minor re-do of work.

Medium Level of consequence: Waste of critical-path time, causing an event that costs a large amount of money to fix.

High Level of consequence:  Causing the failure of the intended purpose of the review document, causing injury (probably death); causing defeat of a safety system, causing damage to the involved equipment, causing a costly nuclear plant trip, causing a power plant loss of power, or causing a safety event that may jeopardize the health and safety of the general public, etc.

If a high-risk omission error would cause a failure of the intended purpose of the document, such as missing a critical requirement that could change the approach of the analysis, the review work shall stop, and the document shall be returned to the preparer immediately to fix the errors. This type of high-risk omission error is called “Review-Stop Error.”

Step 5: Recommend Corrections or Corrective Actions

Each identified Reasoning error shall be corrected in the review document. In this step, each error, except those review-stop-errors, is analyzed for its best possible corrections (or corrective actions).

Step 6: Summarize the Identified Reasoning Errors

All identified Reasoning errors shall be listed in a summary table. For each error, the following Reasoning shall be stated in the summary table for the reason of traceability.

    - Description of Error

    - Type of Error

    - Level of Risk (Review-Stop Risk, High Risk or Low Risk)

    - Method that Identifies the Error

    - Recommended Corrections

Attachment 1

Common Reasoning Errors

Introduction

A reasoning error (R) is an error made in logical thinking, applying logical rules, or establishing logical rules for a specific purpose. These errors fall into six categories: deductive logic errors, inductive logic errors, abductive logic errors, decision‑making errors, judgment errors, and planning errors.

Deductive Logic Errors (R1)

Omission of Rule Coverage (R1.1): Some necessary conditions are not included in the rule.

Lack of Redundancy (R1.2): Critical rules lack redundant verification actions needed to ensure success.

Rule Contradiction (R1.3): Two rules contradict each other, or requirements are not consistently stated (e.g., mixing 'may' or 'should' with 'shall').

Software Programming Error (R1.4):

(R1.4.1) Rules are incorrectly programmed.

(R.1.4.2) Fail to consider race conditions, i.e., timing conflicts between rules.

(R.1.4.3) Insufficient memory

(R.1.4.4) Loops with no exit condition

(R.1.4.5) Outdated software versions

Inductive Logic Errors (R2)

Confirmation Bias (R2.1): Seeking or interpreting information in a way that confirms existing beliefs.

False Cause–Effect Relation (R2.2): Mistaking correlation as evidence of causation.

Inadequate Hypothesis Testing (R2.3): Performing invalid, incomplete, or incorrect hypothesis testing.

Abductive Logic Errors (R3)

Overlooking (R3.1): Failing to consider possible causes.

Confirmation Bias (R3.2): Assuming a preferred belief is the cause without evidence.

Availability Heuristic (R3.3): Choosing the most common or familiar explanation rather than the correct one or collecting only the data available or from familiar sources.

Inadequate Supporting/Refuting Data Collection (R3.4): Failing to gather adequate data to validate or refute causes.

Inadequate Support/Refute Analysis (R3.5): Using an insufficient root‑cause analysis method.

Decision‑Making Errors (R4)

- Decision‑making errors occur when choosing among alternatives based on flawed evaluation.

- Three decision categories: (1) resolving past issues, (2) performing current tasks, (3) selecting.

- a future path for a specific purpose. Decision errors are as follows:

- Inadequate prediction of future environment (R4.1)

- Inadequate evaluation factors (R4.2)

- Inadequate rigor in making high-risk decisions (R4.3)

- Inadequate assigning relative importance to evaluation factors (R4.4)

- Inadequate communication with stakeholders (R4.5)

- Insufficient options, i.e., less than 5 in a high-risk decision making (R4.6)

- Inadequate selection of options (R4.7)

- Poor situation assessment (R4.8)

- Inadequate awareness of cues to revise original decisions (R4.9)

- Wrong corrective action due to wrong cause (R4.10)

- Inadequate layers of protection for single-point vulnerability situations (R4.10).

Judgment Errors (R5)

Judgment Error in judging existence or not (R5.1): Incorrectly determining whether a condition exists.

Judgment Error in Judging Correlation or Not (R5.2): Incorrectly determining whether relationships among parameters exist.

Judgment Error in Judging Randomness or Not (R5.3): Incorrectly judging whether an abnormal condition is random or systematic.

Judgment Error in Judging Cause of Equipment Failures (R5.4): Incorrectly determining whether failure is caused by design, construction, installation, or maintenance issues.

Judgment Error in Judging a Trip (or an Un-safe Condition) or Not (R5.5): A design error that fails to correctly optimize the false positive and false positive rates, considering the consequences of tripping, not tripping, and the development and maintenance cost of the tripping mechanism. The failure rate of tripping when it needs to trip is covered by the false positive (FP) rate. The failure rate of no-tripping when the conditions are safe (no need to trip) is covered by false negative (FN) rate. To minimize false positive/false negative rate, there are many methods as listed in the following table:

| - Scheme | - FP reduction factor vs 1/1 | - FN reduction factor vs 1/1 |
| --- | --- | --- |
| - 1/1 | 1.00 | 1.00 |
| - 1/2 | 0.503 | 100 |
| - 2/2 | 100 | 0.503 |
| - 2/3 | 33.6 | 33.6 |
| - 2/4 | 16.9 | 2,519 |
| - 3/4 | 2,519 | 16.9 |
| - 2/4 Confirm and ¾ trip | 2,519 | 2,519 |
| - 2/4 Confirm and ¾ trip + Confirm by an Independent Info | ~7,000 | ~7,000 |

Planning Errors (R6)

Inadequate Rigor in High-Risk Planning (R6.1)

Inadequate Establishment of Command and Control for Plan Execution (R6.2)

Inadequate Identification of Necessary Work and Support Items (R6.3)

Inadequate Sequencing Work Items (R6.4)

Critical Path Items Incorrectly Identified (R6.5)

Inadequate Preparation Work for Critical Path Items (R6.6)

Inadequate Layers of Protection for Critical Assumptions (R6.7)

Inadequate Exit Planning, Including Exit Criteria, etc. (R6.8)

Inadequate Prioritization of High-Risk Work Items (R6.9)

Inadequate Parallelization of Work to Speed Up Work (R6.10)

Inadequate Use of Advanced Technology to Improve Planning (R6.11)

Inadequate Contingency Planning for the Worst (R6.12)

Inadequate Use of Different Approaches to Minimize Unnecessary Work (R6.13)

Attachment 2

39 Methods to Find Commission Errors

In a document, the way to find commission, “should-do but done wrong”, errors are to analyze the descriptions, explanations, analyses, discussions, conclusions, and interpretations to see whether they violate commonly accepted rules, requirements, principles, formulas, or phenomena. If any are violated, then a “should-do but done wrong” error exists.

1.0 Violating Regulations

1.1 Violating industry rules related to the procedure.

1.2 Violating local government regulations for the industry.

1.3 Violating national government regulations for industry.

1.4 Violating laws.

1.5 Violating company management regulations.

2.0 Violating Requirements

2.1 Company strategy violates trend requirements.

2.2 Company strategy violates general customer requirements.

3.0 Violating Logic

3.1 Violating deductive logic.

3.2 Violating inductive logic.

3.3 Violating abductive (causal reasoning) logic.

3.4 Violating mathematical logic (e.g., Boolean logic)

4.0 Violating Mathematical Formulas

4.1 Violating principles and formulas of mathematical operations.

4.2 Violating geometric principles and formulas.

4.3 Violating algebraic principles and formulas.

4.5 Violating calculus principles and formulas.

5.0 Violating Scientific Principles

5.1 Violating physical principles and formulas.

5.2 Violating chemical principles and formulas.

5.3 Violating biological principles and formulas.

5.4 Violating engineering mechanics principles and formulas.

5.5 Violating earth and space science principles and formulas.

5.6 Violating social science principles and formulas.

5.7 Violating scientific testing principles and formulas.

5.8 Violating scientific analysis principles.

6.0 Violating Statistics (Probability) Principles

6.1 Violating conditional statistics principles and formulas.

6.2 Violating regression statistics principles and formulas.

6.3 Violating inferential statistics principles and formulas.

7.0 Violating Social Ethical Principles

7.1 Violating commonly accepted fairness principles.

7.2 Violating commonly accepted integrity principles.

7.3 Violating commonly accepted humanitarian principles.

8.0 Finding Errors from Phenomena

8.1 “Anomaly Method”: Analyze an abnormal event; often, the cause is one (or a group of) errors (e.g., incorrect work instructions, wrong tools).

8.2 “Balance Method”: Analyze two different things with similar goals, functions, or processes. If one has certain items the other lacks, the imbalance may be caused by errors.

8.3 “Continuity Method”: Analyze sequential procedural steps or descriptions. If continuity is broken, an error likely exists.

8.4 “Difference Method”: Compare similar things (processes, standards, products) and identify differences. These differences may be caused by errors.

8.5 “Expectation Method”: If something does not meet expectations or standards, analyze the reason. The cause may be an error.

8.6 “Independent Verification Method”: Redo the analysis, calculation, or procedure using another method or team. If results differ, the original may contain errors.

8.7 “Cause–Effect Relationship Method”: A valid causal relationship must include cause, process, and result. Missing any of these indicates an error.

8.8 “Causal Probability Method”: In root-cause analysis, if the probability of the identified root cause is lower than the probability of the event, the root cause analysis contains errors.

8.9 “Experience Method”: Use past experience to identify possible errors in documents.